# STATE OF CONNECTICUT

# DEPARTMENT OF ENERGY & ENVIRONMENTAL PROTECTION

# OFFICE OF ENVIRONMENTAL REVIEW

# 79 ELM STREET, HARTFORD, CT 06106-5127

To:	Eric McPhee - Supervising Sanitary Engineer

DPH - Drinking Water Section, 450 Capitol Avenue, MS#51WAT, Hartford

From:	David J. Fox - Senior Environmental Analyst	Telephone:   860-424-4111

Date:	May 8, 2014	E-Mail:  david.fox@ct.gov

Subject:	Water Main, Marlborough

The Department of Energy & Environmental Protection (DEEP) has received the Notice of Scoping for the proposed construction of a public water supply system to serve an area in the center of Marlborough.  The following comments are submitted for your consideration.

The source of water for this project has not been identified (there are no major suppliers in the vicinity).  Obviously, the potential for new or expanded wells in the vicinity to be affected by the existing water quality issues being addressed by the project would have to be considered.

If the withdrawal of groundwater from one or more wells joined in one system has the capacity to yield more than 50,000 gallons per day (gpd), then a permit will be required from the Inland Water Resources Division (IWRD) pursuant to section 22a-368 of the Connecticut General Statutes (CGS) for the diversion of waters of the State.  Wells with a capacity of no greater than 250,000 gpd from either a stratified drift or bedrock aquifer may be eligible for a general permit (DEP-IWRD-GP-012).  A diversion is authorized under this general permit upon receipt by the DEEP of a complete and sufficient request for authorization and subsequent issuance to the requester of a formal letter of authorization from the DEEP.  For further information, contact the division at 860-424-3019.  Fact sheets regarding IWRD permit programs and permit application forms can be downloaded at: IWRD Permits.

The Natural Resources Conservation Service’s Soil Survey depicts a band of Ridgebury, Leicester & Whitman extremely stony soils, a regulated wetland soil, associated with a drainageway running southeasterly from School Street.  It is unknown whether the main will be installed under the roadway or shoulders, with no direct wetland impacts, or beyond previously filled areas.  If there are any undeveloped areas within the area to be impacted, it is recommended that a certified soil scientist perform a reconnaissance of the site in order to determine whether there are any areas which would be regulated as wetlands or watercourses as defined by section 22a-38 (15) and (16) of the CGS, respectively.  If the reconnaissance identifies regulated areas, they should be delineated.  Any inland wetlands or watercourses at the site are regulated by the local inland wetlands agency, pursuant to section 22a-42 of the CGS.  Many local agencies have established setback or buffer areas and require review and approval of activities within these upland areas adjacent to wetlands or watercourses.  The local agency should be contacted regarding permit requirements.

In order to protect wetlands and watercourses on and adjacent to the site, strict erosion and sediment controls should be employed during construction.  The Connecticut Guidelines for Soil Erosion and Sediment Control prepared by the Connecticut Council on Soil and Water Conservation in cooperation with DEEP is a recommended source of technical assistance in the selection and design of appropriate control measures.  The 2002 revised edition of the Guidelines is available online at: Erosion Control Guidelines.

If the water main is to be tested and disinfected, the discharge would be covered by the General Permit for the Discharge of Hydrostatic Pressure Testing Wastewater (DEP-PERD-GP-011).  This general permit applies to all discharges of waters used to test the structural integrity of new or used tanks and pipelines that hold or transfer drinking water, sewage, or natural gas.  The general permit contains pH, chlorine, oil and grease, and suspended solids limits which will need to be complied with during the testing and verified through monitoring.  Registration is required to be submitted to the Department in order for the discharges to be authorized by this general permit.  A fact sheet, the general permit which includes the registration form, titled Notice of Coverage, and the Application Transmittal form may be downloaded at: Hydrostatic GP

Stormwater discharges from construction sites where one or more acres are to be disturbed, regardless of project phasing, require a permit from the Permitting & Enforcement Division.  The General Permit for the Discharge of Stormwater and Dewatering Wastewaters Associated with Construction Activities (DEEP-WPED-GP-015) will cover these discharges.  For projects disturbing five or more acres, registration describing the site and the construction activity must be submitted to the Department prior to the initiation of construction.  A stormwater pollution control plan, including measures such as erosion and sediment controls and post construction stormwater management, must be prepared.

The construction stormwater general permit dictates separate compliance procedures for Locally Approvable projects and Locally Exempt projects (as defined in the permit).  Locally Exempt construction projects disturbing over 1 acre must submit a registration form and Stormwater Pollution Control Plan (SWPCP) to the Department.  Locally Approvable construction projects with a total disturbed area of one to five acres are not required to register with the Department provided the development plan has been approved by a municipal land use agency and adheres to local erosion and sediment control land use regulations and the CT Guidelines for Soil Erosion and Sediment Control.  Locally Approvable construction projects with a total disturbed area of five or more acres must submit a registration form to the Department.  This registration shall include a certification by a Qualified Professional who designed the project and a certification by a Qualified Professional or regional Conservation District who reviewed the SWPCP and deemed it consistent with the requirements of the general permit.  The SWPCP for Locally Approvable projects is not required to be submitted to the Department unless requested.  For further information, contact the division at 860-424-3018.  A copy of the general permit as well as registration forms may be downloaded at: Construction Stormwater GP.

Development plans for utilities in urban areas that entail soil excavation should include a protocol for sampling and analysis of potentially contaminated soil.  A soil management plan should be developed for the project to deal with soils during construction.  The Department’s Guidance for Utility Company Excavation should be used a guide in developing the plan.  The guidance is available on-line at: Utility Guidance .

The Natural Diversity Data Base (NDDB), maintained by DEEP, does not anticipate negative impacts to any Federally listed endangered or threatened species or species listed by the State, pursuant to section 26-306 of the CGS, as endangered, threatened or special concern in the project area.  This determination is good for one year.  Please re-submit an NDDB Request for Review if the scope of work changes or if work has not begun on this project by May 2015.

Natural Diversity Data Base information includes all information regarding critical biological resources available at the time of the request.  This information is a compilation of data collected over the years by the Department of Energy and Environmental Protection’s Natural History Survey and cooperating units of DEEP, private conservation groups and the scientific community.  This information is not necessarily the result of comprehensive or site-specific field investigations.  Consultations with the Data Base should not be substitutes for on-site surveys required for environmental assessments.  Current research projects and new contributors continue to identify additional populations of species and locations of habitats of concern, as well as, enhance existing data.  Such new information is incorporated into the Data Base as it becomes available.  The result of this review does not preclude the possibility that listed species may be encountered on site and that additional action may be necessary to remain in compliance with certain state permits.

Thank you for the opportunity to review this project.  If there are any questions concerning these comments, please contact me.

cc:	Robert Hannon, DEEP/OPPD

Corrine Fitting, DEEP/WPSD

Dawn McKay, DEEP/NDDB