# Checklist for the Use of Beneficially Reclaimed Materials in Large-Scale Filling Pilot Projects

# (Large-Scale Beneficial Filling Pilot Program)

| Applicant Name: |  |  |
| --- | --- | --- |
| Mailing Address: |  |  |
| City/Town: | State: | Zip Code: |
| Business Phone: | ext.: |  |
| Contact Person: | Phone: | ext.: |
| *E-mail: |  |  |

*By providing an e-mail address you are agreeing to receive official correspondence from DEEP, at this electronic address, concerning the subject application. Please remember to check your security settings to be sure you can receive e-mails from “ct.gov” addresses. Also, please notify DEEP if your e-mail address changes.

The following information is required for the Application for Use of Beneficially Reclaimed Materials in Large-Scale Pilot Filling Programs   Part VI: Supporting Documents to use beneficially reclaimed materials as defined in section 22a-209f(c)(1)(A) of the Connecticut General Statutes (CGS).

Check the box preceding each document to indicate that it is included with this checklist. Unless specifically noted, all supporting documents listed must be included and submitted with this checklist in a paper format and an electronic format (e.g., raw data in Excel spreadsheet, Geographic Information System (GIS) for mapping data)

The following detailed maps (numbers 2 through 4) must be produced, minimally, at a scale of one inch equal to five hundred feet, and must show the North arrow, topography at a contour interval no greater than ten feet and all pertinent features, noting natural and artificial features, within a minimum one-half mile radius of the boundaries of the area: (1) An area map of the site showing, at a minimum, the following physical features: - all existing and proposed above-ground and underground man-made structures and installations; - all roads, railways and other transportation corridors; - all property boundaries and names and addresses of adjacent property owners and all landowners as identified on the latest assessor's map; - all adjacent land uses as identified on the latest existing land use maps; - all rights of way, easements, or other interests on the land upon which the filling area is located, including but not limited to all such interests held by gas, sewer, water, and electric utilities; and - subregional watershed delineations. (2) A water resources map of the site which includes, at a minimum, identification of: - all wetlands, tidal wetlands, watercourses, and other waters as defined in CGS sections 22a-38(15), 22a-29(2), 22a-38(16) and 22a-367, respectively and all waters of the United States as defined in 33 CFR 328.2; - all floodplains as defined in CGS section 25-68b;

Checklist (continued)

- all coastal areas as defined in CGS section 22a-93 and 22a-94; - the surface water quality classification goal(s) of all surface waters as identified on maps adopted pursuant to CGS section 22a-426; - the groundwater quality classification goal(s) of all groundwaters as identified on maps adopted pursuant to CGS section 22a-426; and - all aquifer protection areas as defined in CGS section 22a-354(h)(10). (3) A bedrock geology map of the site which includes identification of the type of bedrock, location of outcrops, bedrock and surface topography, strike and dip of bedding planes or foliation, fault lines, fractures, and any other structural features. (4) Detailed site map(s) at a scale of one inch equal to no greater than one hundred feet with topographic contours at an interval of two or five feet, showing the North arrow, and the entire filling area. The site map(s) shall include identification of the following: - existing topographic contours; - proposed final topographic contours; - all existing and proposed test pit, boring, and observation and monitoring well locations; - all existing and proposed surface water monitoring locations; - locations of proposed fill limits including location of fill limit markers; - all existing and proposed above-ground and underground man-made structures and installations, including but not limited to on-site buildings, fences, gates, and roads; - locations of property boundaries, types (e.g., residential, commercial, municipal) of adjacent property, buffer zones and screening; - all rights of way, easements, or other interests on the land upon which the filling area and its groundwater zone of influence are located, including but not limited to all such interests held by gas, sewer, water, and electric utilities; - the areal extent of the point of discharge to a surface water body; - the existing and potential elevations of the groundwater table at the site; - identification of the location of cross-sections prepared in accordance with paragraph (8) of this checklist; and - the type and location of all existing and proposed sedimentation and erosion controls. (5) Detailed site map(s) at a scale and topographic contour interval equal to the maps required by paragraph (6) of this checklist showing the North arrow, the entire filling area, and all monitoring locations. The site map(s) shall include identification of the following: - Contours of surface water elevations at each surface water monitoring location and groundwater elevations and corresponding point data at each monitoring location for each measurement period and for each hydrogeologic stratum in which groundwater elevations are measured. Elevations shall be based on a minimum of two sets of measurements from all surface water monitoring stations, available wells, test pits, springs, and borings. Measurements shall be taken during spring high water levels, unless otherwise approved by the Department of Energy and Environmental Protection (DEEP).

Checklist (continued)

(6) A minimum of four intersecting cross-sections through the site (including two lines parallel and two lines perpendicular to the general direction of groundwater flow) for each phase of site operations. Each cross-section shall document the stratigraphic variation across the site, and depict the following at a horizontal scale equal to the maps required by paragraph (6) of this checklist: - The three phases of site operations include: existing or pre-preparation site conditions, post-preparation site conditions and proposed final grades after filling activity (cross sections shall include grades, proposed depth of fill material throughout the site, and orientation); - For wells or test borings placed to determine groundwater depth, flow direction, and to provide information to construct the cross-sections required by (8)(A); logs that shall include: location (Latitude and Longitude or other comparable survey location), depth to groundwater, and depth to refusal or total well depth; - maximum and minimum recorded elevations of the groundwater table across each cross-section; - vertical and horizontal direction of groundwater flow within and across each cross-section; - depth of the existing and predicted groundwater zone of influence; and - bedrock features and surface topography. (7) A detailed report describing the projected water quality impacts of the discharge. The report shall include, at a minimum, the following: - a summary of the history identifying past and present land uses at the site and other historical sources or releases resulting in impacts (potential or actual) to the quality of surface and groundwater at the site and adjacent to the site. Such summary shall include, but need not be limited to, a description of past and present land uses at the site and at properties adjacent to the site, identification and addresses of present owners of adjacent property, identification of easements on the site, previous site improvements, identification of the types of materials and wastes stored or disposed of on-site, analysis of aerial photographs of the site, and a synopsis of the purpose, methods and results of any previous environmental investigations or conditions at the site; - a statement by each affected utility identified on the area map(s) that the proposed filling activity protects these utilities in compliance with applicable standards; - a description of the hydrogeologic interaction at the site between the surficial and bedrock geology, the groundwater flow, surface water and the point of discharge; - all boring logs, construction and development details, and other supporting documentation associated with monitor wells installed for the purpose of preparing the hydrogeologic descriptions. Prior to preparation of the application, you may wish to consult with DEEP for information about monitor well installation; - a compilation of data on the quality of groundwater and surface water entering or adjacent to the site for the purposes of characterizing ambient water quality at the site. Such compilation shall include, at a minimum, the following: - samples taken from (1) surface water monitoring locations upstream and downstream from the filling area; (2) monitoring wells located at or adjacent to the site in areas which are or which could potentially be affected by the discharge from the filling area; and (3) a representative sample of all water supply wells within a one-quarter mile of the boundaries of the existing or proposed filling area provided such representative sample shall be chosen in accordance with a sampling program which identifies the number and location of wells and discussion of why such a sampling program is representative of groundwater quality for such area; - a minimum of two sampling events, one month apart, shall be taken from all monitoring locations, monitoring wells, and water supply wells described pursuant to paragraph (8)(F)(i) of this checklist;

Checklist (continued)

- parameters listed at the end of this checklist shall be used to characterize the discharge and its impact on water quality; and - all existing water quality data including, at a minimum, quality assurance and quality control protocols for the collection and analysis of groundwater and surface water samples, the analytical methods used and their method detection limits. The results of all such water quality data shall be presented on graphs which depict parameter concentrations on the ordinate and sampling events on the abscissa for each monitoring location and each pertinent hydrogeologic stratum. - A detailed discussion of the possible impact of the existing and/or proposed discharge in terms of the conformance of such discharge with the Connecticut Water Quality Standards (i.e., RCSA section(s) 22a-426-1 to 9) and water quality criteria published pursuant to the Clean Water Act section 304(a). Such discussion shall be prepared in accordance with the following: - For discharges to groundwater, evaluate such impact for the bedrock and unconsolidated deposits at the site boundary, adjacent wetlands, and surface waters, and existing or potential water supply wells. The evaluation shall be based on the water quality characterization described in paragraph (8)(E) of this checklist and the estimate of the quantity of the existing and proposed discharge volumes calculated for the area. In evaluating such impacts, the applicant shall assume that the underlying geologic formations do not have any capacity to physically, chemically, or biologically alter, retard, or attenuate the water quality impacts of the discharge. For water quality impacts from the discharge, the applicant shall consider the average existing ambient groundwater and surface water quality for each parameter. Predicted surface water impacts shall be estimated using seven-day, ten-year low flows obtained from stage measurements or calculated from the United States Geological Survey Connecticut Basin Reports or an equivalent low flow for areas affected by tidal exchange or which are flow regulated; and - For discharges to a surface water, evaluate such impact based on the water quality characterization described in paragraph (8)(E) of this checklist and the estimate of the quantity of the existing and proposed discharge volumes calculated for the area. For water quality impacts from the discharge, the applicant shall consider the average ambient surface water quality for each parameter. Predicted surface water impacts shall be estimated using seven-day, ten-year low flows obtained from stage measurements or calculated from the United States Geological Survey Connecticut Basin Reports or an equivalent low flow for areas affected by tidal exchange or which are flow regulated. (8) A Placement Plan suitable for field use by the operator for the construction, operation, and management of the filling area. Such plan shall ensure that the filling area is operated and maintained to: minimize the size of the working face of such area; minimize erosion, run-on, the infiltration of stormwater and maximize stormwater runoff. At a minimum, such plan shall include a description of the following: - area, volume of expected fill and expected site life; - types of materials proposed to be accepted as described in the Material Acceptability Protocol, methods of measuring and monitoring fill materials, the monitoring parameters, schedule and provisions for the inspection as materials are deposited at the working face; - details and sequence of construction of the filling area; - orientation, sequence and construction of lifts and cells which minimizes the infiltration of stormwater; - location, dimensions, construction details of access roads, and a description of traffic flows; - type and amount of equipment needed for operations;

Checklist (continued)

- area, volume of expected fill and expected site life; - types of materials proposed to be accepted as described in the Material Acceptability Protocol, methods of measuring and monitoring fill materials, the monitoring parameters, schedule and provisions for the inspection as materials are deposited at the working face; - details and sequence of construction of the filling area; - orientation, sequence and construction of lifts and cells which minimizes the infiltration of stormwater; - location, dimensions, construction details of access roads, and a description of traffic flows; - type and amount of equipment needed for operations; - the estimated number and responsibilities of staff to ensure operational compliance; - daily operations including: volumes/tonnage of received fill material, other information to be recorded and recording procedures; controls necessary to protect public health, safety and welfare; site security which may include fencing, signage and video surveillance; dust and odor controls; fire protection, emergency procedures, communications equipment, regular maintenance schedules; - a system of dispersion berms, trenches or other drainage structures to prevent stormwater infiltration and to direct water away from the fill area; and - a stormwater discharge system that includes sedimentation and erosion controls capable of containing a 24-hour, 25-year design storm. A stormwater discharge system is subject to the permit requirements of CGS section 22a-430 and RCSA sections 22a-430-3 and 22a-430-4. (9) A plan for a ground and surface water quality monitoring program, including but not limited to quality assurance and quality control protocols for the collection of all surface water and groundwater samples, a listing of the parameters to be monitored at each monitoring location, their respective analytical methods and method detection limits, precipitation hydrographs for the site, and a schedule for performing and reporting the results of such monitoring program to DEEP. Such program shall be designed to evaluate whether the groundwater zone of influence is within the right of possession of the property owner, and that the discharge will conform with the Water Quality Standards and water quality criteria published pursuant to the Clean Water Act section 304(a). (10) A plan for a monthly inspection and monitoring program of the perimeter and side slopes of the fill area, the banks of surface waters, and any wetlands adjacent to the area to identify the degree and extent of seeps. At a minimum the following shall be included as part of such plan: - All persistent seeps shall be identified, sampled and analyzed for the parameters listed at the end of the checklist, and any other substances necessary to characterize the discharge and its impact on water quality. Such sample collection and analyses shall be performed in accordance with the requirements of paragraph (12) of this checklist. Persistent seeps are defined as active discharges, which have been identified at any one location in three consecutive inspections events; and - submittal of a report which includes, at a minimum, a map drawn to a scale of one inch equal to one hundred feet showing the location and extent of all seeps and which describes the chemical composition of the leachate seeps, any sampling results, the discharge rate, and which includes a plan for remediation of such seeps and a schedule for carrying out the monthly inspection plan.

Checklist (continued)

(11) A Redevelopment Plan for the fill area which includes, but is not limited to: a description of the proposed development and site use upon completion of filling and closure of the site, engineering plans and drawings in support of such redevelopment, provisions for the grading of slopes, placement of final cover, stabilization with soils and/or vegetation to minimize erosion, run-on/run-off, and infiltration. (12) A detailed plan for long-term maintenance and monitoring of the site that shall address at a minimum: - monitoring of water quality in the area of the site for 30 years; - inspection, monitoring, maintenance of: the site, the fill area and cap, treatment systems or facilities, integrity of the groundwater monitoring wells, etc.; and - identification of all persons that will be legally responsible for the site following filling and closure of site. Place a check mark preceding each of the following to verify that the requirements have been met. Yes NO Is the proposed [filling area or site] consistent with the criteria in the latest Connecticut Water Quality Standards (i.e., RCSA sections 22a-426-1 to 22a-426-9)? Yes NO Does the applicant have the right of possession, by means of fee interest or easement, to the zone of influence of the filling area? Yes NO Is the zone of influence of the filling area located, in whole or in part, within an aquifer protection area as defined in CGS section 22a-354(h)(10)? Yes NO Is there any potable water supply well located within the zone of influence of the [filling area or site?], or located such that recharge of such well from the zone of influence could be induced by pumpage? Yes NO Is the [filling area or site?] located, in whole or in part, within any tidal wetlands as defined under CGS section 22a-29?

# Parameters

Parameters include the following and Appendix I of Part 258, which is attached. Upon review of your application, you may be required to analyze the discharge for additional parameters identified in Appendix II of Part 258, which is also attached.

If you have any questions, please contact ...

Common Name

Total Dissolved Solids

Total Suspended Solids

Alkalinity

pH

Total Dissolved Iron

Total Dissolved Manganese

Ammonia (expressed as Nitrogen)

Nitrate (expressed as Nitrogen)

Sodium

Potassium

Chlorides

Sulfates

Oxidation - Reduction Potential

Biological Oxygen Demand (5 day)

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| Inorganic Constituents: |  |  |  |  |
| Antimony | (Total) | Antimony | 6010 | 300 |
|  |  |  | 7040 | 2000 |
|  |  |  | 7041 | 30 |
| Arsenic | (Total) | Arsenic | 6010 | 500 |
|  |  |  | 7060 | 10 |
|  |  |  | 7061 | 20 |
| Barium | (Total) | Barium | 6010 | 20 |
|  |  |  | 7080 | 1000 |
| Beryllium | (Total) | Beryllium | 6010 | 3 |
|  |  |  | 7090 | 50 |
|  |  |  | 7091 | 2 |
| Cadmium | (Total) | Cadmium | 6010 | 40 |
|  |  |  | 7130 | 50 |
|  |  |  | 7131 | 1 |
| Chromium | (Total) | Chromium | 6010 | 70 |
|  |  |  | 7190 | 500 |
|  |  |  | 7191 | 10 |
| Cobalt | (Total) | Cobalt | 6010 | 70 |
|  |  |  | 7200 | 500 |
|  |  |  | 7201 | 10 |
| Copper | (Total) | Copper | 6010 | 60 |
|  |  |  | 7210 | 200 |
|  |  |  | 7211 | 10 |
| Lead | (Total) | Lead | 6010 | 400 |
|  |  |  | 7420 | 1000 |
|  |  |  | 7421 | 10 |
| Nickel | (Total) | Nickel | 6010 | 150 |
|  |  |  | 7520 | 400 |
| Selenium | (Total) | Selenium | 6010 | 750 |
|  |  |  | 7740 | 20 |
|  |  |  | 7741 | 20 |
| Silver | (Total) | Silver | 6010 | 70 |
|  |  |  | 7760 | 100 |
|  |  |  | 7761 | 10 |
| Thallium | (Total) | Thallium | 6010 | 400 |
|  |  |  | 7840 | 1000 |
|  |  |  | 7841 | 10 |
| Vanadium | (Total) | Vanadium | 6010 | 80 |
|  |  |  | 7910 | 2000 |
|  |  |  | 7911 | 40 |
| Zinc | (Total) | Zinc | 6010 | 20 |
|  |  |  | 7950 | 50 |
|  |  |  | 7951 | 0.5 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| Organic Constituents: |  |  |  |  |
| Acetone | 67-64-1 | 2-Propanone | 8260 | 100 |
| Acrylonitrile | 107-13-1 | 2-Propenenitrile | 8030 | 5 |
|  |  |  | 8260 | 200 |
| Benzene | 71-43-2 | Benzene | 8020 | 2 |
|  |  |  | 8021 | 0.1 |
|  |  |  | 8260 | 5 |
| Bromochloromethane; | 74-97-5 | Methane, bromochloro- | 8021 | 0.1 |
| Chlorobromomethane |  |  | 8260 | 5 |
| Bromodichloromethane; | 75-27-4 | Methane, bromodichloro- | 8010 | 1 |
| Dibromochloromethane |  |  | 8021 | 0.2 |
|  |  |  | 8260 | 5 |
| Bromoform; Tribromomethane | 75-25-2 | Methane, tribromo- | 8010 | 2 |
|  |  |  | 8021 | 15 |
|  |  |  | 8260 | 5 |
| Carbon disulfide | 75-15-0 | Carbon disulfide | 8260 | 100 |
| Carbon tetrachloride | 56-23-5 | Methane, tetrachloro- | 8010 | 1 |
|  |  |  | 8021 | 0.1 |
|  |  |  | 8260 | 10 |
| Chlorobenzene | 108-90-7 | Benzene, chloro- | 8010 | 2 |
|  |  |  | 8020 | 2 |
|  |  |  | 8021 | 0.1 |
|  |  |  | 8260 | 5 |
| Chloroethane; Ethyl chloride | 75-00-3 | Ethane, chloro- | 8010 | 5 |
|  |  |  | 8021 | 1 |
|  |  |  | 8260 | 10 |
| Chloroform; Trichloromethane | 67-66-3 | Methane, trichloro- | 8010 | 0.5 |
|  |  |  | 8021 | 0.2 |
|  |  |  | 8260 | 5 |
| Dibromochloromethane; | 124-48-1 | Methane, dibromochloro- | 8010 | 1 |
| Chlorodibromomethane |  |  | 8021 | 0.3 |
|  |  |  | 8260 | 5 |
| 1, 2-Dibromo-3-chloropropane; DBCP | 96-12-8 | Propane, 1,2-dibrome-3-chloro- | 8011 8021 8260 | 0.1 30 25 |
| 1, 2-Dibromoethane; Ethylene | 106-93-4 | Ethane, 1,2-dibromo- | 8011 | 0.1 |
| dribromide; EDB |  |  | 8021 8260 | 10 5 |
| o-Dichlorobenzene; 1, 2- | 95-50-1 | Benzene, 1,2-dichloro- | 8010 | 2 |
| Dichlorobenzene |  |  | 8020 | 5 |
|  |  |  | 8021 | 0.5 |
|  |  |  | 8120 | 10 |
|  |  |  | 8260 | 5 |
|  |  |  | 8270 | 10 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| p-Dichlorobenzene; 1, 4- Dichlorobenzene | 106-46-7 | Benzene, 1,4-dichloro- | 8010 8020 8021 8120 8260 8270 | 2 5 0.1 15 5 10 |
| trans-1, 4-Dichloro-2-butene | 110-57-6 | 2-Butene, 1,4-dichloro-, (E)- | 8260 | 100 |
| 1, 1-Dichloroethane; Ethylidene chloride | 75-34-3 | Ethane, 1,1-dichloro- | 8010 8021 8260 | 1 0.5 5 |
| 1, 2-Dichloroethane; Ethylene dichloride | 107-06-2 | Ethane, 1,1-dichloro- | 8010 8021 8260 | 0.5 0.3 5 |
| 1, 1-Dichloroethylene; 1, 1- Dichloroethene; Vinylidene chloride | 75-35-4 | Ethene, 1,1-dichloro- | 8010 8021 8260 | 1 0.5 5 |
| cis-1, 2-Dichloroethylene; cis-1, 2- Dichloroethene | 156-59-2 | Ethene, 1,2-dichloro-, (Z)- | 8021 8260 | 0.2 5 |
| trans-1, 2-Dichloroethylene trans- 1, 2-Dichloroethene | 156-60-5 | Ethene, 1,2-dichloro-, (E)- | 8010 8021 8260 | 1 0.5 5 |
| 1, 2-Dichloropropane; Propylene dichloride | 78-87-5 | Propane, 1,2-dichloro- | 8010 8021 8260 | 0.5 0.05 5 |
| cis-1, 3-Dichloropropene | 10061-01-5 | 1-Propene, 1,3-dichloro-, (Z)- | 8010 8260 | 20 10 |
| trans-1, 3-Dichloropropene | 10061-02-6 | 1-Propene, 1,3-dichloro-, (E)- | 8010 8260 | 5 10 |
| Ethylbenzene | 100-41-4 | Benzene, ethyl- | 8020 8221 8260 | 2 0.05 5 |
| 2-Hexanone; Methyl butyl ketone | 591-78-6 | 2-Hexanone | 8260 | 50 |
| Methyl bromide; Bromomethane | 74-83-9 | Methane, bromo- | 8010 8021 | 20 10 |
| Methyl chloride; Chloromethane | 74-87-3 | Methane, chloro- | 8010 8021 | 1 0.3 |
| Methylene bromide; Dibromomethane | 74-95-3 | Methane, dibromo- | 8010 8021 8260 | 15 20 10 |
| Methylene chloride; Dichloromethane | 75-09-2 | Methane, dichloro- | 8010 8021 8260 | 5 0.2 10 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| Methyl ethyl ketone; MEK; 2- Butanone | 78-93-3 | 2-Butanone | 8015 8260 | 10 100 |
| Methyl iodide; Iodomethane | 74-88-4 | Methane, iodo- | 8010 8260 | 40 10 |
| 4-Methyl-2-pentanone; Methyl isobutyl ketone | 108-10-1 | 2-Pentanone, 4-methyl- | 8015 8260 | 5 100 |
| Styrene | 100-42-5 | Benzene, ethenyl- | 8020 8021 8260 | 1 0.1 10 |
| 1, 1, 1, 2-Tetrachloroethane | 630-20-6 | Ethane, 1,1,1,2-tetrachloro- | 8010 8021 8260 | 5 0.05 5 |
| 1, 1, 2, 2-Tetrachloroethane | 79-34-5 | Ethane, 1,1,2,2-tetrachloro- | 8010 8021 8260 | 0.5 0.1 5 |
| Tetrachloroethylene; Tetrachloroethene; Perchloroethylene | 127-18-4 | Ethene, tetrachloro- | 8010 8021 8260 | 0.5 0.5 5 |
| Toluene | 108-88-3 | Benzene, methyl- | 8020 8021 8260 | 2 0.1 5 |
| 1, 1, 1-Trichloroethane; Methylchloroform | 71-55-6 | Ethane, 1,1,1-trichloro- | 8010 8021 8260 | 0.3 0.3 5 |
| 1, 1, 2-Trichloroethane | 79-00-5 | Ethane, 1,1,2-trichloro- | 8010 8260 | 0.2 5 |
| Trichloroethylene; Trichloroethene | 79-01-6 | Ethene, trichloro- | 8010 8021 8260 | 1 0.2 5 |
| Trichlorofluoromethane; CFC-11 | 75-69-4 | Methane, trichlorofluoro- | 8010 8021 8260 | 10 0.3 5 |
| 1, 2, 3-Trichloropropane | 96-18-4 | Propane, 1,2,3-trichloro- | 8010 8021 8260 | 10 5 15 |
| Vinyl acetate | 108-05-4 | Acetic acid, ethenyl ester | 8260 | 50 |
| Vinyl chloride; Chloroethene | 75-01-4 | Ethene, chloro- | 8010 8021 8260 | 2 0.4 10 |
| Xylenes | 1330-20-7 | Benzene, dimethyl- | 8020 8021 8260 | 5 0.2 5 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| Acenaphthene | 83-32-9 | Acenaphthylene, 1,2-dihydro- | 8100 8270 | 200 10 |
| Acenaphthylene | 208-96-8 | Acenaphthylene | 8100 8270 | 200 10 |
| Acetone | 67-64-1 | 2-Propanone | 8260 | 100 |
| Acetonitrile; Methyl cyanide | 75-05-8 | Acetonitrile | 8015 | 100 |
| Acetophenone | 98-86-2 | Ethanone, 1-phenyl | 8270 | 10 |
| 2-Acetylaminofluorene; 2-AAF | 53-96-3 | Acetamide, N-9H-fluoren-2-yl | 8270 | 20 |
| Acrolein | 107-02-8 | 2-Propenal | 8030 8260 | 5 100 |
| Acrylonitrile | 107-13-1 | 2-Propenenitrile | 8030 8260 | 5 200 |
| Aldrin | 309-00-2 | 1, 4:5, 8-Dimethanonaphthalene, 1, 2, 3, 4, 10, 10-hexachloro-1, 4, 4a, 5, 8, 8a- hexahydro- (1a, 4a, 4ab, 5a, 8a, 8ab) | 8080 8270 | 0.05 10 |
| Allyl chloride | 107-05-1 | 1-Propene, 3-chloro | 8010 8260 | 5 10 |
| 4-Aminobipherryl | 92-67-1 | [1, 11-Biphenyl]-4-amine | 8270 | 20 |
| Anthracene | 120-12-7 | Anthracene | 8100 8270 | 200 10 |
| Antimony | (Total) | Antimony | 6010 7040 7041 | 300 2000 30 |
| Arsenic | (Total) | Arsenic | 6010 7060 7061 | 500 10 20 |
| Barium | (Total) | Barium | 6010 7080 | 20 1000 |
| Benzene | 71-43-2 | Benzene | 8020 8021 8260 | 2 0.1 5 |
| Benzo[a]anthracene; Benzanthracene | 56-55-3 | Benz[a]anthracene | 8100 8270 | 200 10 |
| Benzo[b]fluoranthene | 205-99-2 | Benz[e]acephenanthrylene | 8100 8270 | 200 10 |
| Benzo[k]fluoranthene | 207-08-9 | Benzo[k]fluoranthene | 8100 8270 | 200 10 |
| Benzo[ghi]perylene | 191-24-2 | Benzo[ghi]perylene | 8100 8270 | 200 10 |
| Benzo[a]pyrene | 50-32-8 | Benzo[a]pyrene | 8100 8270 | 200 10 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| Benzyl alcohol | 100-51-6 | Benzenemethanol | 8270 | 20 |
| Beryllium | (Total) | Beryllium | 6010 7090 7091 | 3 50 2 |
| alpha-BHC | 319-84-6 | Cyclohexane, 1, 2, 3, 4, 5, 6-hexachloro- , (1a, 2a, 3b, 4a, 5b, 6b) | 8080 8270 | 0.05 10 |
| beta-BHC | 319-85-7 | Cyclohexane, 1, 2, 3, 4, 5, 6-hexachloro- , (1a, 2b, 3a, 4b, 5a, 6b) | 8080 8270 | 0.05 20 |
| delta-BHC | 319-86-8 | Cyclohexane, 1, 2, 3, 4, 5, 6-hexachloro- , (1a, 2a, 3a, 4b, 5a, 6b) | 8080 8270 | 0.1 20 |
| gamma-BHC; Lindane | 58-89-9 | Cyclohexane, 1,2,3,4,5,6-hexachloro-, (1a,2a,3b,4a,5a,6b) | 8080 8270 | 0.05 20 |
| Bis(2-chloroethoxy)methane | 111-91-1 | Ethane, 1,11-[methylenebis(oxy)]bis[2- chloro- | 8110 8270 | 5 10 |
| Bis(2-chloroethyl)ether; Dichloroethyl ether | 111-44-4 | Ethane, 1,11-oxybis[2-chloro- | 8110 8270 | 3 10 |
| Bis-(2-chloro-1-methylethyl) ether; 2, 21 - Dichlorodiisopropyl ether; DCIP, see Note 7 | 108-60-1 | Propane, 2,21-oxybis[1-chloro- | 8110 8270 | 10 10 |
| Bis(2-ethylhexyl) phthalate | 117-81-7 | 1,2-Benzenedicarboxylic acid, bis(2- ethylhexyl) ester | 8060 | 20 |
| Bromochloromethane; Chlorobromomethane | 74-97-5 | Methane, bromochloro- | 8021 8260 | 0.1 5 |
| Bromodichloromethane; Dibromochloromethane | 75-27-4 | Methane, bromodichloro- | 8010 8021 8260 | 1 0.2 5 |
| Bromoform; Tribromomethane | 75-25-2 | Methane, tribromo- | 8010 8021 8260 | 2 15 5 |
| 4-Bromophenyl phenyl ether | 101-55-3 | Benzene, 1-bromo-4-phenoxy- | 8110 8270 | 25 10 |
| Butyl benzyl phthalate; Benzyl butyl phthalate | 85-68-7 | 1,2-Benzenedicarboxylic acid, butyl phenylmethyl ester | 8060 8270 | 5 10 |
| Cadmium | (Total) | Cadmium | 6010 7130 7131 | 40 50 1 |
| Carbon disulfide | 75-15-0 | Carbon disulfide | 8260 | 100 |
| Carbon tetrachloride | 56-23-5 | Methane, tetrachloro- | 8010 8021 8260 | 1 0.1 10 |
| Chlordane | See Note 8 | 4,7-Methano-1H-indene, 1,2,4,5,6,7,8,8- octachloro-2,3,3a,4,7,7a-hexahydro- | 8080 8270 | 0.1 50 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| p-Chloroaniline | 106-47-8 | Benzenamine, 4-chloro- | 8270 | 20 |
| Chlorobenzene | 108-90-7 | Benzene, chloro- | 8010 8020 8021 8260 | 2 2 0.1 5 |
| Chlorobenzilate | 510-15-6 | Benzeneacetic acid, 4-chloro-a-(4- chlorophenyl)-a-hydroxy-, ethyl ester | 8270 | 10 |
| p-Chloro-m-cresol; 4-Chloro-3- methylphenol | 59-50-7 | Phenol, 4-chloro-3-methyl- | 8040 8270 | 5 20 |
| Chloroethane; Ethyl chloride | 75-00-3 | Ethane, chloro- | 8010 8021 8260 | 5 1 10 |
| Chloroform; Trichloromethane | 67-66-3 | Methane, trichloro- | 8010 8021 8260 | 0.5 0.2 5 |
| 2-Chloronaphthalene | 91-58-7 | Naphthalene, 2-chloro- | 8120 8270 | 10 10 |
| 2-Chlorophenol | 95-57-8 | Phenol, 2-chloro- | 8040 8270 | 5 10 |
| 4-Chlorophenyl phenyl ether | 7005-72-3 | Benzene, 1-chloro-4-phenoxy- | 8110 8270 | 40 10 |
| Chloroprene | 126-99-8 | 1,3-Butadiene, 2-chloro- | 8010 8260 | 50 20 |
| Chromium | (Total) | Chromium | 6010 7190 7191 | 70 500 10 |
| Chrysene | 218-01-9 | Chrysene | 8100 8270 | 200 10 |
| Cobalt | (Total) | Cobalt | 6010 7200 7201 | 70 500 10 |
| Copper | (Total) | Copper | 6010 7210 7211 | 60 200 10 |
| m-Cresol; 3-methylphenol | 108-39-4 | Phenol, 3-methyl- | 8270 | 10 |
| o-Cresol; 2-methylphenol | 95-48-7 | Phenol, 2-methyl- | 8270 | 10 |
| p-Cresol; 4-methylphenol | 106-44-5 | Phenol, 4-methyl- | 8270 | 10 |
| Cyanide | 57-12-5 | Cyanide | 9010 | 200 |
| 2, 4-D; 2, 4- Dichlorophenoxyacetic acid | 94-75-7 | Acetic acid, (2,4-dichlorophenoxy)- | 8150 | 10 |
| 4, 41 - DDD | 72-54-8 | Benzene 1,11-(2,2- dichloroethylidene)bis[4-chloro- | 8080 8270 | 0.1 10 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| 4, 41 - DDE | 72-55-9 | Benzene 1,11- (dichloroethyenylidene)bis[4-chloro- | 8080 8270 | 0.05 10 |
| 4, 41 - DDT | 50-29-3 | Benzene 1,11-(2,2,2- trichloroethylidene)bis[4-chloro- | 8080 8270 | 0.1 10 |
| Diallate | 2303-16-4 | Carbamothioic acid, bis(1-methylethyl)- S-(2,3-dichloro-2-pro-penyl) ester | 8270 | 10 |
| Dibenz [a, h] anthracene | 53-70-3 | Dibenz [a, h] anthracene | 8100 8270 | 200 10 |
| Dibenzofuran | 132-64-9 | Dibenzofuran | 8270 | 10 |
| Dibromochloromethane; Chlorodibromomethane | 124-48-1 | Methane, dibromochloro- | 8010 8021 8260 | 1 0.3 5 |
| 1, 2-Dibromo-3-chloropropane; DBCP | 96-12-8 | Propane, 1,2-dibrome-3-chloro- | 8011 8021 8260 | 0.1 30 25 |
| 1, 2-Dibromoethane; Ethylene dribromide; EDB | 106-93-4 | Ethane, 1,2-dibromo- | 8011 8021 8260 | 0.1 10 5 |
| Di-n-butyl phthalate | 84-74-2 | 1,2-Benzenedicarboxylic acid, dibutyl ester | 8060 8270 | 5 10 |
| o-Dichlorobenzene; 1, 2- Dichlorobenzene | 95-50-1 | Benzene, 1,2-dichloro- | 8010 8020 8021 8120 8260 8270 | 2 5 0.5 10 5 10 |
| m-Dichlorobenzene; 1, 3- Dichlorobenzene | 541-73-1 | Benzene, 1,3-Dichloro- | 8010 8020 8021 8120 8260 8270 | 5 5 0.2 10 5 10 |
| p-Dichlorobenzene; 1, 4- Dichlorobenzene | 106-46-7 | Benzene, 1,4-dichloro- | 8010 8020 8021 8120 8260 8270 | 2 5 0.1 15 5 10 |
| 3, 31- Dichlorobenzidine | 91-94-1 | [1,11-Biphenyl]-4,41-diamine, 3,31- dichloro- | 8270 | 20 |
| trans-1, 4-Dichloro-2-butene | 110-57-6 | 2-Butene, 1,4-dichloro-, (E)- | 8260 | 100 |
| Dichlorodifluoromethane; CFC 12 | 75-71-8 | Methane, dichlorodifluoro- | 8021 8260 | 0.5 5 |
| 1, 1-Dichloroethane; Ethyldidene chloride | 75-34-3 | Ethane, 1,1-dichloro- | 8010 8021 8260 | 1 0.5 5 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| 1, 2-Dichloroethane; Ethylene dichloride | 107-06-2 | Ethane, 1,1-dichloro- | 8010 8021 8260 | 0.5 0.3 5 |
| 1, 1-Dichloroethylene; 1, 1- Dichloroethene; Vinylidene chloride | 75-35-4 | Ethene, 1,1-dichloro- | 8010 8021 8260 | 1 0.5 5 |
| cis-1, 2-Dichloroethylene; cis-1, 2- Dichloroethene | 156-59-2 | Ethene, 1,2-dichloro-, (Z)- | 8021 8260 | 0.2 5 |
| trans-1, 2-Dichloroethylene trans- 1, 2-Dichloroethene | 156-60-5 | Ethene, 1,2-dichloro-, (E)- | 8010 8021 8260 | 1 0.5 5 |
| 2, 4-Dichlorophenol | 120-83-2 | Phenol, 2,4-dichloro- | 8040 8270 | 5 10 |
| 2, 6-Dichlorophenol | 87-65-0 | Phenol, 2,6-dichloro- | 8270 | 10 |
| 1, 2-Dichloropropane; Propylene dichloride | 78-87-5 | Propane, 1,2-dichloro- | 8010 8021 8260 | 0.5 0.05 5 |
| 1, 3-Dichloropropane; Trimethylene dichloride | 142-28-9 | Propane, 1,3-dichloro- | 8021 8260 | 0.3 5 |
| 2, 2-Dichloropropane; Isopropylidene chloride | 594-20-7 | Propane, 2,2-dichloro- | 8021 8260 | 0.5 15 |
| 1, 1-Dichloropropene | 563-58-6 | 1-Propene, 1,1-dichloro- | 8021 8260 | 0.2 5 |
| cis-1, 3-Dichloropropene | 10061-01-5 | 1-Propene, 1,3-dichloro-, (Z)- | 8010 8260 | 20 10 |
| trans-1, 3-Dichloropropene | 10061-02-6 | 1-Propene, 1,3-dichloro-, (E)- | 8010 8260 | 5 10 |
| Dieldrin | 60-57-1 | 2,7:3,6-Dimethanonaphth[2,3-b]oxirene, 3,4,5,6,9,9-hexa,chloro- 1a,2,2a,3,6,6a,7,7a-octahydro-, (1aa,2b,2aa,3b,6b,6aa,7b,7aa)- | 8080 8270 | 0.05 10 |
| Diethyl phthalate | 84-66-2 | 1,2-Benzenedicarboxylic acid, diethyl ester | 8060 8270 | 5 10 |
| 0, 0-Diethyl 0-2-pyrazinyl phosphorothioate; Thionazin | 297-97-2 | Phosphorothioic acid, 0,0-diethyl 0- pyrazinyl ester | 8141 8270 | 5 20 |
| Dimethoate | 60-51-5 | Phosphorodithioic acid, 0,0-dimethyl S- [2-(methylamino)-2-oxoethyl] ester | 8141 8270 | 3 20 |
| p-(Dimethylamino)azobenzene | 60-11-7 | Benzenamine, N,N-dimethyl-4- (phenylazo)- | 8270 | 10 |
| 7, 12-Dimethylbenz [a] anthracene | 57-97-6 | Benz[a]anthracene, 7,12-dimethyl- | 8270 | 10 |
| 3, 31-Dimethylbenzidine | 119-93-7 | [1,11-Biphenyl]-4,41-diamine, 3,31- dimethyl- | 8270 | 10 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| 2, 4-Dimethylphenol; m-Xylenol | 105-67-9 | Phenol, 2,4-dimethyl- | 8040 8270 | 5 10 |
| Dimethyl phthalate | 131-11-3 | 1,2-Benzenedicarboxylic acid, dimethyl ester | 8060 8270 | 5 10 |
| m-Dinitrobenzene | 99-65-0 | Benzene, 1,3-dinitro- | 8270 | 20 |
| 4, 6-Dinitro-o-cresol 4, 6-Dinitro-2- methylphenol | 534-52-1 | Phenol, 2-methyl-4,6-dinitro | 8040 8270 | 150 50 |
| 2, 4-Dinitrophenol | 51-28-5 | Phenol, 2,4-dinitro- | 8040 8270 | 150 50 |
| 2, 4-Dinitrotoluene | 121-14-2 | Benzene, 1-methyl-2,4-dinitro- | 8090 8270 | 0.2 10 |
| 2, 6-Dinitrotoluene | 606-20-2 | Benzene, 2-methyl-1,3-dinitro- | 8090 8270 | 0.1 10 |
| Dinoseb; DNBP; 2-sec-Butyl-4,6- dinitrophenol | 88-85-7 | Phenol, 2-(1-methylpropyl)-4,6-dinitro- | 8150 8270 | 1 20 |
| Di-n-octyl phthalate | 117-84-0 | 1,2-Benzenedicarboxylic acid, dioctyl ester | 8060 8270 | 30 10 |
| Diphenylamine | 122-39-4 | Benzenamine, N-phenyl- | 8270 | 10 |
| Disulfoton | 298-04-4 | Phosphorodithioic acid, 0,0-diethyl S-[2- (ethylthio)ethyl] ester | 8140 8141 8270 | 2 0.5 10 |
| Endosulfan I | 959-98-8 | 6,9-Methano-2,4,3-benzodioxathiepin, 6,7,8,9,10,10-hexachloro-1,5,5a,6,9,9a- hexahydro-, 3-oxide | 8080 8270 | 0.1 20 |
| Endosulfan II | 33213-65-9 | 6,9-Methano-2,4,3-benzodioxathiepin, 6,7,8,9,10,10-hexachloro-1,5,5a,6,9,9a- hexahydro-, 3-oxide, (3a,5aa,6b,9b,9aa)- | 8080 8270 | 0.05 20 |
| Endosulfan sulfate | 1031-07-8 | 6,9-Methano-2,4,3-benzodioxathiepin, 6,7,8,9,10,10-hexachloro-1,5,5a,6,9,9a- hexahydro-, 3-3-dioxide | 8080 8270 | 0.5 10 |
| Endrin | 72-20-8 | 2,7:3,6-Dimethanonaphth[2,3-b]oxirene, 3,4,5,6,9,9-hexachloro- 1a,2,2a,3,6,6a,7,7a-octahydro-, (1aa,2b,2ab,3a,6a,6ab,7b,7aa)- | 8080 8270 | 0.1 20 |
| Endrin aldehyde | 7421-93-4 | 1,2,4-Methenocyclopenta[cd]pentalene- 5-carboxaldehyde,2,2a,3,3,4,7- hexachlorodecahydro-, (1a,2b,2ab,4b,4ab,5b,6ab,6bb,7r*)- | 8080 8270 | 0.2 10 |
| Ethylbenzene | 100-41-4 | Benzene, ethyl- | 8020 8221 8260 | 2 0.05 5 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| Ethyl methacrylate | 97-63-2 | 2-Propenoic acid, 2-methyl-, ethyl ester | 8015 8260 8270 | 5 10 10 |
| Ethyl methanesulfonate | 62-50-0 | Methanesulfonic acid, ethyl ester | 8270 | 20 |
| Famphur | 52-85-7 | Phosphorothioic acid, 0-[4- [(dimethylamino)sulfonyl]phenyl]0,0- dimethyl ester | 8270 | 20 |
| Fluoranthene | 206-44-0 | Fluoranthene | 8100 8270 | 200 10 |
| Fluorene | 86-73-7 | 9H-Fluorene | 8100 8270 | 200 10 |
| Heptachlor | 76-44-8 | 4,7-Methano-1H-indene, 1,4,5,6,7,8,8- heptachloro-3a,4,7,7a-tetrahydro- | 8080 8270 | 0.05 10 |
| Heptachlor epoxide | 1024-57-3 | 2,5-Methano-2H-indeno[1,2-b]oxirene, 2,3,4,5,6,7,7-heptachloro- 1a,1b,5,5a,6,6a-hexahydro-, (1aa,1bb,2a,5a,5ab,6b,6aa) | 8080 8270 | 1 10 |
| Hexachlorobenzene | 118-74-1 | Benzene, hexachloro- | 8120 8270 | 0.5 10 |
| Hexachlorobutadiene | 87-68-3 | 1,3-Butadiene, 1,1,2,3,4,4-hexachloro- | 8021 8120 8260 8270 | 0.05 5 10 10 |
| Hexachlorocyclopentadiene | 77-47-4 | 1,3-Cyclopentadiene, 1,2,3,4,5,5- hexachloro- | 8120 8270 | 5 10 |
| Hexachloroethane | 67-72-1 | Ethane, hexachloro- | 8120 8260 8270 | 0.5 10 10 |
| Hexachloropropene | 1888-71-7 | 1-Propene, 1,1,2,3,3,3-hexachloro- | 8270 | 10 |
| 2-Hexanone; Methyl butyl ketone | 591-78-6 | 2-Hexanone | 8260 | 50 |
| Indeno (1, 2, 3-cd) pyrene | 193-39-5 | Indeno(1,2,3-cd)pyrene | 8100 8270 | 200 10 |
| Isobutyl alcohol | 78-83-1 | 1-Propanol, 2-methyl- | 8015 8240 | 50 100 |
| Isodrin | 465-73-6 | 1,4,5,8- Dimethanonaphthalene,1,2,3,4,10,10- hexachloro-1,4,4a,5,8,8a hexahydro- (1a,4a,4ab,5b,8b,8ab)- | 8270 8260 | 20 10 |
| Isophorone | 78-59-1 | 2-Cyclohexen-1-one, 3,5,5-trimethyl- | 8090 8270 | 60 10 |
| Isosafrole | 120-58-1 | 1,3-Benzodioxole, 5-(1-propenyl)- | 8270 | 10 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| Kepone | 143-50-0 | 1,3,4-Metheno-2H-cyclobuta[cd]pentalen- 2-one,1,1a,3,3a,4,5,5,5a,5b,6- decachlorooctahydro- | 8270 | 20 |
| Lead | (Total) | Lead | 6010 7420 7421 | 400 1000 10 |
| Mercury | (Total) | Mercury | 7470 | 2 |
| Methacrylonitrile | 126-98-7 | 2-Propenenitrile, 2-methyl- | 8015 8260 | 5 100 |
| Methapyrilene | 91-80-5 | 1,2-Ethanediamine, N,N-dimethyl-N1-2- pyridinyl-N1/2-thienyl-methyl)- | 8270 | 100 |
| Methoxychlor | 72-43-5 | Benzene,1,11- (2,2,2,trichloroethylidene)bis[4-methoxy- | 8080 8270 | 2 10 |
| Methyl bromide; Bromomethane | 74-83-9 | Methane, bromo- | 8010 8021 | 20 10 |
| Methyl chloride; Chloromethane | 74-87-3 | Methane, chloro- | 8010 8021 | 1 0.3 |
| 3-Methylcholanthrene | 56-49-5 | Benz[j]aceanthrylene, 1,2-dihydro-3- methyl- | 8270 | 10 |
| Methyl ethyl ketone; MEK; 2- Butanone | 78-93-3 | 2-Butanone | 8015 8260 | 10 100 |
| Methyl iodide; Iodomethane | 74-88-4 | Methane, iodo- | 8010 8260 | 40 10 |
| Methyl methacrylate | 80-62-6 | 2-Propenoic acid, 2-methyl-, methyl ester | 8015 8260 | 2 30 |
| Methyl methanesulfonate | 66-27-3 | Methanesulfonic acid, methyl ester | 8270 | 10 |
| 2-Methylnaphthalene | 91-57-6 | Naphthalene, 2-methyl- | 8270 | 10 |
| Methyl parathion; Parathion methyl | 298-00-0 | Phosphorothioic acid, 0,0-dimethyl 0-(4- nitrophenyl) ester | 8140 8141 8270 | 0.5 1 10 |
| 4-Methyl-2-pentanone; Methyl isobutyl ketone | 108-10-1 | 2-Pentanone, 4-methyl- | 8015 8260 | 5 100 |
| Methylene bromide; Dibromomethane | 74-95-3 | Methane, dibromo- | 8010 8021 8260 | 15 20 10 |
| Methylene chloride; Dichloromethane | 75-09-2 | Methane, dichloro- | 8010 8021 8260 | 5 0.2 10 |
| Naphthalene | 91-20-3 | Naphthalene | 8021 8100 8260 8270 | 0.5 200 5 10 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| 1, 4-Naphthoquinone | 130-15-4 | 1,4-Naphthalenedione | 8270 | 10 |
| 1-Naphthylamine | 134-32-7 | 1-Naphthalenamine | 8270 | 10 |
| 2-Naphthylamine | 91-59-8 | 2-Naphthalenamine | 8270 | 10 |
| Nickel | (Total) | Nickel | 6010 7520 | 150 400 |
| o-Nitroaniline; 2-Nitroaniline | 88-74-4 | Benzenamine, 2-nitro- | 8270 | 50 |
| m-Nitroaniline; 3-Nitroanile | 99-09-2 | Benzenamine, 3-nitro- | 8270 | 50 |
| p-Nitroaniline; 4-Nitroaniline | 100-01-6 | Benzenamine, 4-nitro- | 8270 | 20 |
| Nitrobenzene | 98-95-3 | Benzene, nitro- | 8090 8270 | 40 10 |
| o-Nitrophenol; 2-Nitrophenol | 88-75-5 | Phenol, 2-nitro- | 8040 8270 | 5 10 |
| p-Nitrophenol; 4-Nitrophenol | 100-02-7 | Phenol, 4-nitro- | 8040 8270 | 10 50 |
| N-Nitrosodi-n-butylamine | 924-16-3 | 1-Butanamine, N-butyl-N-nitroso- | 8270 | 10 |
| N-Nitrosodiethylamine | 55-18-5 | Ethanamine, N-ethyl-N-nitroso- | 8270 | 20 |
| N-Nitrosodimethylamine | 62-75-9 | Methanamine, N-methyl-N-nitroso- | 8070 | 2 |
| N-Nitrosodiphenylamine | 86-30-6 | Benzenamine, N-nitroso-N-phenyl- | 8070 | 5 |
| N-Nitrosodipropylamine; N- Nitroso-N-dipropylamine; Di-n- pro-pylnitrosamine | 621-64-7 | 1-Propanamine, N-nitroso-N-propyl- | 8070 | 10 |
| N-Nitrosomethylethalamine | 10595-95-6 | Ethanamine, N-methyl-N-nitroso- | 8270 | 10 |
| N-Nitrosopiperidine | 100-75-4 | Piperidine, 1-nitroso- | 8270 | 20 |
| N-Nitrosopyrrolidine | 930-55-2 | Pyrrolidine, 1-nitroso- | 8270 | 40 |
| 5-Nitro-o-toluidine | 99-55-8 | Benzenamine, 2-methyl-5-nitro- | 8270 | 10 |
| Parathion | 56-38-2 | Phosphorothioic acid, 0,0-diethyl 0-(4- nitrophenyl) ester | 8141 8270 | 0.5 10 |
| Pentachlorobenzene | 608-93-5 | Benzene, pentachloro- | 8270 | 10 |
| Pentachloronitrobenzene | 82-68-8 | Benzene, pentachloronitro- | 8270 | 20 |
| Pentachlorophenol | 87-86-5 | Phenol, pentachloro- | 8040 8270 | 5 50 |
| Phenacetin | 62-44-2 | Acetamide, N-(4-ethoxyphenl) | 8270 | 20 |
| Phenanthrene | 85-01-8 | Phenanthrene | 8100 8270 | 200 10 |
| Phenol | 108-95-2 | Phenol | 8040 | 1 |
| p-Phenylenediamine | 106-50-3 | 1,4-Benzenediamine | 8270 | 10 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| Phorate | 298-02-2 | Phosphorodithioic acid, 0,0-diethyl S- [(ethylthio)methyl] ester | 8140 8141 8270 | 2 0.5 10 |
| Polychlorinated biphenyls; PCBs; Aroclors | See Note 9 | 1,11-Biphenyl, chloro derivatives | 8080 8270 | 50 200 |
| Pronamide | 23950-58-5 | Benzamide, 3,5-dichloro-N-(1,1-dimethyl- 2-propynyl)- | 8270 | 10 |
| Propionitrile; Ethyl cyanide | 107-12-0 | Propanenitrile | 8015 8260 | 60 150 |
| Pyrene | 129-00-0 | Pyrene | 8100 8270 | 200 10 |
| Safrole | 94-59-7 | 1,3-Benzodioxole, 5-(2-propenyl)- | 8270 | 10 |
| Selenium | (Total) | Selenium | 6010 7740 7741 | 750 20 20 |
| Silver | (Total) | Silver | 6010 7760 7761 | 70 100 10 |
| Silvex; 2, 4, 5-TP | 93-72-1 | Propanoic acid, 2-(2,4,5- trichlorophenoxyl)- | 8150 | 2 |
| Styrene | 100-42-5 | Benzene, ethenyl- | 8020 8021 8260 | 1 0.1 10 |
| Sulfide | 18496-25-8 | Sulfide | 9030 | 4000 |
| 2, 4, 5-T; 2, 4, 5- Trichlorophenoxyacetic acid | 93-76-5 | Acetic acid, (2,3,5-trichlorophenoxy)- | 8150 | 2 |
| 1, 2, 4, 5-Tetrachlorobenzene | 95-94-3 | Benzene, 1,2,4,5-tetrachloro- | 8270 | 10 |
| 1, 1, 1, 2-Tetrachloroethane | 630-20-6 | Ethane, 1,1,1,2-tetrachloro- | 8010 8021 8260 | 5 0.05 5 |
| 1, 1, 2, 2-Tetrachloroethane | 79-34-5 | Ethane, 1,1,2,2-tetrachloro- | 8010 8021 8260 | 0.5 0.1 5 |
| Tetrachloroethylene; Tetrachloroethene; Perchloroethylene | 127-18-4 | Ethene, tetrachloro- | 8010 8021 8260 | 0.5 0.5 5 |
| 2, 3, 4, 6-Tetrachlorophenol | 58-90-2 | Phenol, 2,3,4,6-tetrachloro- | 8270 | 10 |
| Thallium | (Total) | Thallium | 6010 7840 7841 | 400 1000 10 |
| Tin | (Total) | Tin | 6010 | 40 |
| Toluene | 108-88-3 | Benzene, methyl- | 8020 8021 8260 | 2 0.1 5 |

| Common Name | CAS RN | Chemical Abstracts Service Index Name | Suggested methods | PQL (µg/L) |
| --- | --- | --- | --- | --- |
| o-Toluidine | 95-53-4 | Benzenamine, 2-methyl- | 8270 | 10 |
| Toxaphene | See Note 10 | Toxaphene | 8080 | 2 |
| 1, 2, 4-Trichlorobenzene | 120-82-1 | Benzene, 1,2,4-trichloro- | 8021 8120 8260 8270 | 0.3 0.5 10 10 |
| 1, 1, 1-Trichloroethane; Methylchloroform | 71-55-6 | Ethane, 1,1,1-trichloro- | 8010 8021 8260 | 0.3 0.3 5 |
| 1, 1, 2-Trichloroethane | 79-00-5 | Ethane, 1,1,2-trichloro- | 8010 8260 | 0.2 5 |
| Trichloroethylene; Trichloroethene | 79-01-6 | Ethene, trichloro- | 8010 8021 8260 | 1 0.2 5 |
| Trichlorofluoromethane; CFC-11 | 75-69-4 | Methane, trichlorofluoro- | 8010 8021 8260 | 10 0.3 5 |
| 2, 4, 5-Trichlorophenol | 95-95-4 | Phenol, 2,4,5-trichloro- | 8270 | 10 |
| 2, 4, 6-Trichlorophenol | 88-06-2 | Phenol, 2,4,6-trichloro- | 8040 8270 | 5 10 |
| 1, 2, 3-Trichloropropane | 96-18-4 | Propane, 1,2,3-trichloro- | 8010 8021 8260 | 10 5 15 |
| 0, 0, 0-Triethyl phosphorothioate | 126-68-1 | Phosphorothioic acid, 0,0,0-triethylester | 8270 | 10 |
| sym-Trinitrobenzene | 99-35-4 | Benzene, 1,3,5-trinitro- | 8270 | 10 |
| Vanadium | (Total) | Vanadium | 6010 7910 7911 | 80 2000 40 |
| Vinyl acetate | 108-05-4 | Acetic acid, ethenyl ester | 8260 | 50 |
| Vinyl chloride; Chloroethene | 75-01-4 | Ethene, chloro- | 8010 8021 8260 | 2 0.4 10 |
| Xylene (total) | See Note 11 | Benzene, dimethyl- | 8020 8021 8260 | 5 0.2 5 |
| Zinc | (Total) | Zinc | 6010 7950 7951 | 20 50 0.5 |

Notes:

- The regulatory requirements pertain only to the list of substances; the right hand columns (Methods and PQL) are given for informational purposes only. See also footnotes 5 and 6.

- Common names are those widely used in government regulations, scientific publications, and commerce; synonyms exist for many chemicals.

- Chemical Abstracts Service registry number. Where “Total” is entered, all species in the groundwater that contain this element are included.

- CAS index are those used in the 9th Collective Index.

- Suggested Methods refer to analytical procedure numbers used in EPA Report SW-846 “Test Methods for Evaluating Solid Waste”, third edition, November 1986, as revised, December 1987. Analytical details can be found in SW-846 and in documentation on file at the agency. CAUTION: The methods listed are representative SW-846 procedures and may not always be the most suitable method(s) for monitoring an analyte under the regulations.

- Practical Quantitation Limits (PQLs) are the lowest concentrations of analytes in groundwaters that can be reliably determined within specified limits of precision and accuracy by the indicated methods under routine laboratory operating conditions. The PQLs listed are generally stated to one significant figure. PQLs are based on 5 ml samples for volatile organics and 1 L samples for semivolatile organics. CAUTION: The PQL valves in many cases are based only on a general estimate for the method and not on a determination for individual compounds; PQLs are not a part of the regulation.

- This substance is often called Bis(2-chloroisopropyl) ether, the name Chemical Abstracts Service applies to its noncommercial isomer, Propane, 2,2”-o.xybis [2-chloro- (CAS RN 39638-32-9)].

- Chlordane: This entry includes alpha-chlordane (CAS RN 5103-71-9), beta-chlordane (CAS RN 5103- 74-2), gammachlordane (CAS RN 5566-34-7), and constituents of chlordane (CAS RN 57-74-9 and CAS RN 12789-03-6). PQL shown is for technical chlordane. PQLs of specific isomers are about 20 µg/L by method 8270.

- Polychlorinated biphenyls (CAS RN 1336-36-3); this category contains congener chemicals, including constituents of Aroclor 1016 (CAS RN 12674-11-2), Aroclor 1221 (CAS RN 11104-28-2), Aroclor 1232

(CAS RN 11141-16-5), Aroclor 1242 (CAS RN 53469-21-9), Aroclor 1248 (CAS RN 12672-29-6),

Aroclor 1254 (CAS RN 11097-69-1), and Aroclor 1260 (CAS RN 11096-82-5). The PQL shown is an average for PCB congeners.

- Toxaphene: This entry includes congener chemicals contained in technical toxaphene (CAS RN 8001- 35-2), i.e., chlorinated camphene.

- Xylene (total): This entry includes o-xylene (CAS RN 96-47-6), m-xylene (CAS RN 108-38-3), p-xylene (CAS RN 106-42-3), and unspecified xylenes (dimethylbenzenes) (CAS RN 1330-20-7). PQLs for method 8021 are 0.2 for o-xylene and 0.1 for m- or p-xylene. The PQL for m-xylene is 2.0 µg/L by method 8020 or 8260.