OEP/EPC ICM PowerPoint (Quick) Check List   Rev: Jan. 2023

Information should be kept focused on Permitting/Regulated items.

Site Info:

- Include location map, site photos, inlet/outlet, and upstream & downstream photos

- Include “Project Purpose and Need”, if necessary to justify need for project

Existing/Proposed Bridge Info:

- Describe bridge opening distance/clear span (not to be confused with bridge length), for existing and proposed structures

- Provide in an elevation view of proposed bridge (with existing shown lightly, if possible) and longitudinal view.  Include elevations: OHW, 100 yr, etc as appropriate for project.  In longitudinal view, depict where any proposed channel grading will match into existing channel.

- Provide length and slope of pipe for fisheries knowledge (if applicable)

- Provide BFW & Openness ratio; note whether meets 1.2 BFW, prepare to answer questions if not met

- Existing and proposed freeboard (as defined by regulators, from design WSE to low point on roadway at the structure); be prepared to answer questions if requirement not met

- Show any proposed riprap placement

- Provide flow rate information & drainage area, include if existing & proposed pass required design storm (as applicable)

- Include location of access roads (can be schematic/approx..), if needed, and identify if temporary or permanent

- Include proposed water handling concept for project construction; minimum hydraulic opening

- Be prepared for questions regarding project alternatives.  Designer should have info/justifications for discussion

Regulated Areas:

- Show OHW, wetland limits, floodplain/floodway limits (as applicable). Call-out/Bold.  Colored lines are preferred

- Show OHW & 100 yr in elevation view

- Depict wetland/watercourse impacts (value in SF)

- Depict floodplain/floodway impacts (value in CY), only need permanent impact value

- W.S.E. - Any water surface elevation increase or decrease? (have inundation maps avail if needed, to be ready to answer)

- Coastal show CJL, HTL, MHW, MLW, tidal wetland boundaries

- Coastal include impacts below CJL and HTL

Resource Coordination:  note if any requirements are being met/incorporated

- Check if EFH (and any Time of Year restrictions?), ESA

- Fisheries comments received? Any enhancements incorporated? Time of Year restrictions?

- NDDB, any issue? Conclusion?

- USFWS IPac, present results

- SHPO concerns/determination

OEP/EPC ICM PowerPoint (Quick) Check List – (continued)

Other Information:

- Provide anticipated permits (verify SV criteria met), and associated GP

- Provide project schedule (desired construction start and number of seasons)

- Provide project contact information

- Don’t need tree clearing, don’t need erodible area

- Don’t need detour map (unless pertinent for a justification)

- Don’t need to show roadway profiles (unless pertinent for a justification ex. freeboard)

NOTE:  More detailed information regarding the Interagency Coordination Meeting and the PowerPoint Presentations can be found in the CTDOT Bureau of Policy and Planning – Office of Environmental Planning’s webpage under the Environmental Permitting Unit section.

Environmental Planning, Permitting & Compliance (ct.gov)