ITEM #1113550A - DETECTOR CABLE (OPTICAL)

SYSTEM DESCRIPTION:

The emergency vehicle traffic signal priority control system shall enable designated vehicles to remotely cause the traffic signal controller to advance to and/or hold a desired traffic signal display by using existing controller functions. The control shall be effective for a range of 12M (40 feet) to 548M ( 1,800 feet ) along an unobstructed "line of sight" path.

The system shall consist of the following components:

A.	Vehicle Emitter:  mounted on the emergency vehicle shall transmit optical energy signals only in the forward direction.  Optical emitters must be capable of activating other major manufacturers’ optical detectors.

B.	Phase Selector:  shall cause the signal controller to advance to and/or hold the desired traffic signal display for the emergency vehicle. A pre-emption system chassis shall house two phase selectors.

C.	Optical Detector:  mounted on or near a traffic signal shall receive the optical energy signals generated by the vehicle emitter.  Optical detectors must be capable of receiving other major manufacturers’ optically emitted signals.

1.	Detector (Type A)   1 Direction, 1 Channel

2.	Detector (Type B)   2 Direction, 1 Channel

3.	Detector (Type C)   2 Direction, 2 Channel

D.	Detector Cable (Optical).

System Component:

D.   Detector Cable (Optical)

1.	3 Conductor cable with shield and ground wire.

2.	AWG #20  (7x28) stranded.

3.	Individually tinned copper strands.

4.	Conductor insulation: 600 volt , 75 deg. C (167 F.).

5.	1 conductor yellow, 1 conductor blue, 1 conductor orange.

6.	Aluminized mylar  shield tape or equivalent.

7.	AWG #20  (7x28) stranded uninsulated drain wire

8.	DC resistance not to exceed 11.0 ohms per 305M(1000 feet).

9.	Capacitance from one conductor to other two conductors and shield not to exceed 157pf/M (48 pf./ft.).

10.	Jacket: 600 volts, 80 deg. C (176 F.), minimum average wall thickness - 1.14mm (.045").

11.	Finished O.D.:  7.62mm (0.3") max.

Construction Methods:

All equipment except the vehicle emitter assembly shall be installed and wired in a neat and orderly manner in conformance with the manufacturers’ instructions. Detector cables shall be installed continuous with no splices between the optical detector and the AEC.

Method of Measurement:

Detector Cable (Optical) will be measured by the number of meters (linear feet) supplied, installed and accepted.

Basis of Payment:

Payment for Detector Cable (Optical) will include the item unit cost, including the cost of installation.

Pay Item		Pay Unit

=================                       	===========

Detector Cable (Optical)		M(L.F.)

s:\traffic\1406\signal specs\specs\1113550A-detector cable (optical).doc